SURFACE MIX BURNER		GROUP L-14 Linear Burner

......Machine Mounting......				Range: SMDJ

Rectangular Flame Matrix.

Surface Mix.

Oxygen-Gas.

50 - 200 Flame Length x 2 Row

Typical glass industry applications include:

Fire - Flame Polishing and Edge/Rim glazing

of all tabletop and beer glasses.

Various shaping, forming and annealing

operations during the secondary heating of

glass articles, containers and components.

Supply pressure requirements.

Natural gas:       20 - 1000 mbar

Propane gas:     20 - 1000 mbar

Hydrogen:        350 - 1000 mbar  For Hard glass working.

Oxygen:   1 - 4 bar

Cat.No.		Type.			A	B	C	          D - Inlet Size.	kW/hr.

L350		SMDJ-50		50	57	25		1/8”BSP		9.95

L351		SMDJ-75		75	82	40		1/8”BSP		14.90

L352		SMDJ-100		100	107	50		1/8”BSP		19.50

L353		SMDJ-150		150	157	75		¼”BSP		29.50

L354		SMDJ-200		200	207	100		¼”BSP		39.30

Gas and Oxygen flow control valves are available as an optional extra.

To avoid burner face sooting, liquified natural gas [lng] and propane may need a small injection of process air into the burner gas feed. Partial pre-mixing the fuel gas with oxygen will significantly increase the heat transfer rate. The Rapidflame technical department will assist and advise on the selection and positioning of venturi mixers and flow control valves.

Gas burners and combustion equipment must be installed and used in compliance with all relevant applicable regulations and industry codes of practice.

The Rapidflame policy is one of continuous development and improvement, we therefore reserve the right to change any specification or design without prior notice.